U70 Datalink Module

30x30x3mm

Received Power: 0.18W

Output Power: 0.5/1/2W

8g

PO:

Frequency Range: 410-470 MHz

Protocol: Lora/Transparent/South/Mac/TT450S

Air Baud Rate: 500bps/11000bps/12500bps/15500bps/18000bps

Modulation System: CSS/FSK

App:

Surveying, Robot, Agriculture

Feature:

Small Size, Multiple Protocols, Long Working Range

U702 Datalink Module

17x22x3mm

Received Power: 0.1W

Output Power: 0.025/0.05/0.1W

2g

PO:

Frequency Range: 410-470 MHz/863-870 MHz/902-928 MHz

Protocol: Lora

Air Baud Rate: 500bps/11000bps/12500bps/15500bps/18000bps

Modulation System: CSS

App:

Surveying, Robot, IoT

Feature:

Small Size, Multiple Frequencies, Low Power Consumption

K901

Signal Tracking

GPS  L1C/A, L2P, L2C, L5, L1C

BDS-2   B1I, B2I, B3I

BDS-3   B1I, B3I, B1C, B2a, B2b

GALILEO   E1, E5b, E5a, E5 AltBoC*, E6c*

GLONASS   G1, G2, G3*

SBAS   L1C/A, L5

QZSS   L1C/A, L2C, L5, L1C

NAVIC*   L5

PPP     0.1 m Horizontal; 0.2 m Vertical

RTK positioning accuracy     8 mm + 1 ppm Horizontal

15 mm + 1 ppm Vertical

K902

Channels:1688

RTK positioning accuracy：8 mm + 1 ppm Horizontal

15 mm + 1 ppm Vertical

PPP Positioning Accuracy:  0.1 m Horizontal; 0.2 m Vertical

Signal Tracking

GPS   L1C/A, L2P, L2C, L5, L1C

BDS-2    B1I, B2I, B3I

BDS-3    B1I, B3I, B1C, B2a, B2b

GALILEO    E1, E5b, E5a, E5 AltBoC*, E6c*

GLONASS   G1, G2, G3*

SBAS    L1C/A, L5

QZSS    L1C/A, L2C, L5, L1C

NAVIC*   L5

K922

Signal Tracking

GPS   L1C/A, L2P, L2C, L5, L1C*

BDS-2   B1I, B2I, B3I

BDS-3   B1I, B3I, B1C*, B2a, B2b*

GALILEO   E1, E5b, E5a, E5 AltBoC*, E6c*

GLONASS   G1, G2, G3*

SBAS*   L1C/A, L5

QZSS   L1C/A, L2C, L5, L1C*

NAVIC*   L5

RTK positioning accuracy：8 mm + 1 ppm Horizontal

15 mm + 1 ppm Vertical

PPP Positioning Accuracy： 0.1 m Horizontal; 0.2 m Vertical

K801:

IoT, UAV, Personnel Tracking

K803:

Surveying, Agriculture, UAV